| Nebraska Renewable Energy Systems WIND GENERATORS FOR RENEWABLE ELECRICITY PRODUCTION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MODEL |  | NRES 650 |  | NRES 1500 | NRES 3300 | NRES 4200 |
| RATED OUTPUT |  | 650W | 1500W |  | 3300W | 4200W |
| RATED WIND SPEED m/s / mph |  | 10.5 / 25 | 10.5 / 25 |  | 10.5 / 25 | 11.5 / 27 |
| PEAK OUTPUT |  | 750W | 1750W |  | 3650W | 4500W |
| CUT IN m/s / mph |  | 2.7 / 6 | 2.7/ 6 |  | 2.7 / 6 | 2.7 / 6 |
| POLES /RPM - 50hz RPM - 60hz |  | 10 / 600 / 720 | 10 / 600 / 720 |  | 16 / 375 / 450 | 16 / 375 / 450 |
| Monthly KWH 10mph / 4.5 m/s PLF % |  | 68 kWh (18%) | 178 kWh (18%) |  | 352 kWh (18%) | 412 kWh (16%) |
| Monthly KWH 12mph / 5.4 m/s PLF (%) |  | 115 kWh (25%) | 278 kWh (25%) |  | 568 kWh (25%) | 685 kWh (24%) |
| ROTOR DIAMETER |  | 2.20m / 7 ft | 3.20m ./ 11 ft |  | 4.65m / 15 ft | 4.90m / 16.2 ft |
| NUMBER OF BLADES |  | 3 | 3 |  | 2 / 3 | 3 |
| BLADE MATERIAL |  | Carbon Fiber composite | Carbon Fiber composite |  | Carbon Fiber composite | Carbon Fiber composite |
| SWEPT AREA |  | 3.7 / 43 Sq.m/ sq.feet | 9.2 / 95 Sq.m / sq.feet |  | 16.4 / 175 Sq.m / sq.feet | 19 / 205 Sq.m / sq.feet |
| LATERAL THRUST |  | 340 nts. | 800 nts |  | 1200 nts | 1490 nts. |
| GOVERNOR |  | Tilt | Angle |  | Angle | Angle |
| GOVERN SPEED |  | 27mph | 27mph |  | 27mph | 27mph |
| GOV. SHUT-DOWN / OPTIONAL STOP |  | Electro- dynamic Switch | Electro- dynamic Switch |  | Electro- dynamic Switch | Electro-dynamic Switch |
| UNIT WEIGHT |  | 21 Kg | 34 Kg |  | 77 Kg. | 89 Kg |
| GENERATOR |  | PM 3 Phase alternator | PM 3 phase alternator |  | PM 3 phase alternator | PM 3 phase alternator |
| VOLTAGES AVAILABLE |  | 12/ 24 / 36 / 48 | 12 - 240 HV/LV |  | 12 - 240 HV/LV | 24 - 240 HV/LV |
| ELECTRONIC CONTROLLER |  | Incl. but separate | Incl. but separate |  | Incl. but separate | Incl. but separate |
| WARRANTY |  | 2 yrs. | 2 yrs. |  | 2 yrs. | 2 yrs. |
| OPERATING LIFE |  | 20 yrs. | 20 yrs. |  | 20 yrs. | 20 yrs. |
| SURVIVAL WIND |  | 55 m/s | 55 m/s |  | 55 m/s | 55 m/s |
| MFR. SUG. ROUTINE MAINTENANCE |  | Annual inspection | Annual inspection |  | Annual inspection | Annual inspection |

708 North Davis, Oakland, Nebraska 68045 www.nebraskawind.com